EQUIPMENT VALIDATION PROTOCOL (IQ / OQ / PQ)

Machining & Production Facility Controls (CNC / VMC Subsystems)
FDA 21 CFR 820.70(g) | ISO 13485:2016 Clause 7.5.2 | CDSCO MDR 2017

| Equipment Name / Type: | ☐ CNC Lathe Line ☐ Vertical Machining Center (VMC) | Protocol Reference #: | VAL-EQP-XXXX-REV1 |
| --- | --- | --- | --- |
| Equipment Model / ID: | [Enter Model & Unique Asset Tag ID] | Installation Location: | [Cleanroom Class ISO 7 / Workshop Area] |
| Validation Stage: | ☐ IQ ☐ OQ ☐ PQ (Stage Multi-Select) | Target Execution Date: | [DD-MMM-YYYY] |

1. Overview & Objective

This validation protocol establishes the documented framework for the Installation Qualification (IQ), Operational Qualification (OQ), and Performance Qualification (PQ) of high-precision machining systems (CNC lathes and VMC milling lines) used in the manufacture of orthopedic implants or medical device components. The objective is to demonstrate that the machining hardware operates reliably, maintains dimensional specifications within micro-tolerances, and adheres to the strict processing controls required by FDA 21 CFR 820.70, ISO 13485, and CDSCO guidelines.

2. Installation Qualification (IQ)

2.1. Utility & Environmental Verification

Verify that the power supply (3-phase AC, voltage stability), pneumatic air pressure lines (bar/psi limits), and localized cleanroom or dust-extraction infrastructure meet the manufacturer's specified machine baseline inputs.

| IQ Parameter / Component | Acceptance Verification Criteria | Actual Observation / Asset Link | Pass / Fail |
| --- | --- | --- | --- |
| 1. Electrical Supply | 415V AC ± 10%, 3-Phase, 50Hz baseline supply check. | [Measured Voltage: ______ V] | ☐ Pass ☐ Fail |
| 2. Pneumatic Pressure | Stable air supply at 6.0 to 7.0 bar with integrated moisture separator trap. | [Measured Pressure: _____ bar] | ☐ Pass ☐ Fail |
| 3. Software / Firmware | CNC/VMC controller software version must match design specification. | [Controller Version: _________] | ☐ Pass ☐ Fail |

3. Operational Qualification (OQ)

3.1. Machine Functional Checklists

Verify emergency stop relays, interlock doors, auto-lubrication intervals, tool changer indexes (ATC), and axis limit switches are fully responsive under static conditions.

Perform empty-run spindle verification cycles tracking spindle speed accuracy (RPM steps) and coordinate positioning accuracy across X, Y, and Z axes using laser interferometer logs or calibrated dial gauges.

| OQ Parameter / Test Cycle | Target Operational Limits | Recorded Value / Data Trace | Pass / Fail |
| --- | --- | --- | --- |
| 1. Spindle Runout & RPM | Spindle runout < 0.005 mm; RPM accuracy within ±1% of program command. | [Runout: ____ mm ¦ Max RPM: ____] | ☐ Pass ☐ Fail |
| 2. ATC / Tool Indexing | Automatic Tool Changer correctly indexes 100 consecutive cycles without alarm. | [Total Index Faults: _________] | ☐ Pass ☐ Fail |
| 3. Interlock / E-Stop | Spindle stops instantly (< 2.0 sec) upon activation of emergency switch or door open. | [Spindle Stop Time: _____ sec] | ☐ Pass ☐ Fail |

4. Performance Qualification (PQ)

PQ requires executing consecutive machining runs using real raw material lots (e.g., Titanium/PEEK medical bar stock) to demonstrate process stability over extended production shifts. A minimum of 3 consecutive batches must be evaluated using standard statistical tooling metrics (Cpk tracking) to prove that critical product dimensions (e.g., implant outer diameters, thread pitches, and surface finishes) stay comfortably within specification parameters under standard load.

| Critical Dimension / Attribute | Specification Lower/Upper Limits | Statistical Process Capability Target | Observed Process Cpk Status |
| --- | --- | --- | --- |
| 1. Critical Feature Diameter | [e.g., Outers: 4.500 mm ± 0.010 mm] | Cpk >= 1.33 (3 consecutive runs) | [Calculated Cpk Run 1-3: ______] |
| 2. Surface Roughness (Ra) | Ra <= 0.40 micrometers (Post-machining raw profile check) | 100% of samples comply across lot | [Measured Ra Range: _________] |

5. Protocol Approval & Closure Sign-off

| Verification Phase Role | Department / Function | Digital / Physical Signature | Date Signed |
| --- | --- | --- | --- |
| Prepared By: | Validation Engineer / Production Team | [Sign Asset Verification] | [Date] |
| Reviewed By: | Engineering / Maintenance Lead | [Sign Machine Baseline Status] | [Date] |
| Approved By: | Head of Quality Assurance (QA) | [Authorize Equipment Release] | [Date] |